| # MTO Geomatics Photogrammetric Survey # 2D CAD .DWG FILE Production Guideline | Ver. 1 2018-04-04 |
| --- | --- |

CONTENTS

1	PURPOSE	1

2	SCOPE	1

3	PROCESS	1

4	DEFINITIONS	1

5	RELATED DOCUMENTS	1

6	FEEDBACK	2

7	APPROVAL AND REVIEW DETAILS	2

8	APPENDIX	2

# PURPOSE

Describes a best practice for converting AutoCAD Civil 3D 2013 format photogrammetric CAD drawings (with civil objects such as 3D survey figures and cogo points and surfaces) into a plain AutoCAD 2-dimensional (all CAD objects at Z=0) equivalent.

# SCOPE

For use by MTO Geomatics staff and photogrammetric mapping Service Providers

# PROCESS

  - The basis for work is the MTO Standard Civil 3D 2016 PH 2D .dwg template (2013 .dwg format)

  - Standard feature lines and symbols are as set out in the PH Legend .dwg

  - Open file and Turn on and Unfreeze all .DWG layers

  - Civil 3D Workspace > OUTPUT tab > Export Civil 3D Drawing.  By default, the output .DWG filename will be the original filename, prefixed by “ACAD-“

  - Open this new ACAD-{filename} drawing.

  - Save the drawing to a new filename, e.g, “FLAT-ACAD-{filename}”to preserve the 3D ACAD-ONLY version of the previous step, as a safety precaution.  Note that layer filters will be removed with this step, but can be reimported if they have been exported from the original drawing.

  - Run “Super Flatten” LISP Routine on this new file.

  - Inspect the flattened drawing for:

- Features elevations at zero

- Visual conformance with standards for linetypes and blocks*

- Conformance with layer standards

  - Note that the system variable “3DPLINELINETYPEON” must be set to “1” so that the MTO custom linetypes such as scalloped and dashed lines are depicted properly.

  - Note that any original PH terrain model will not be retrievable from this flattened drawing. It will have to be retained in the original Civil 3D drawing as a surface object. It can be translated to Bentley InRoads .dtm format using the 2016 Civil Engineering Data Translator Productivity Pack 1 in the Toolbox tab of Civil 3D.

# DEFINITIONS

n/a

# RELATED DOCUMENTS

- Photogrammetry documents (SharePoint library)

- AutoCAD Standards Guide (MTO Online Library)

- IESCAD package

- LISP Routine, “SuperFlatten 2.0c” by Joe Burke

[Source code and explanatory notes]

# FEEDBACK

  - Provide feedback about this document by emailing Peter.Lamb@ontario.ca

# APPROVAL AND REVIEW DETAILS

| Approval and Review | Details |
| --- | --- |
| Approval Authority |  |
| Advisory Committee to Approval Authority |  |
| Approval and Amendment History | Details |
| Original Approval Authority and Date |  |
| Amendment Authority and Date |  |
| Notes |  |

# APPENDIX